Raspberry PI Wireless Headless Configuration Guide

- Remove all USB sticks from computer!

- Insert microSD card in card reader, if prompted for Format, not necessary. Take note of the letter which appears. Might be two different drives.

- Launch Win32DiskImager.

- Extract from WinZip Raspberry PI archive. Lite is without GUI and faster.

- Select the img file to be written, target device in Win32 Disk Imager, then press Write.

- Check again that you have correct device letter selected (usually the one with boot is the correct one).

- Wait until the job is finished. If an error message appears, remove the microSD card from reader and format it again.

- Edit the file wpa_supplicant.conf with your wi-fi settings.

- Copy to boot the two files wpa_supplicant.conf and ssh.

- Remove SD card from laptop. Insert microSD card in Raspberry PI.

- Switch on hot-spot on smartphone if using smartphone hotspot, otherwise make sure wifi network is connected.

- Connect laptop to smartphone hotspot or wifi network.

- Connect Raspberry PI to power.

- Open cmd, launch ipconfig /all.

- Take note of the IP address assigned by the smartphone.

- Open IPScan application. Search for devices in the same network as it was assigned by the smartphone, i.e. if you have 192.168.43.247 assigned search for /24 network addresses 192.168.43.0 – 192.168.43.255.

- Sort the result by ping to obtain the list of computers. The raspberrypi computer should appear.

- Open Git Bash on computer.

- Write ssh pi@192.168.43.232 where the IP is changed to what you have on the IP scan results. Confirm with yes, then put password raspberry.

- If you get a warning message then you should delete the known hosts by the command rm ~/.ssh/known_hosts

- If you are on home WiFi router: Run command sudo apt-get update

- If you are on home WiFi router: Run command sudo apt-get upgrade.

- If you have installed the graphical version of Raspbian and you want to connect from VNC, run the following command sudo apt-get install realvnc-vnc-server realvnc-vnc-viewer

- Run the command vncserver.

- Launch the realvnc viewer program and type the address of the raspberry pi.

- Username is pi. Password is raspberry.